Start — Edison wakes up and gets ready to run the program.

Clear clap detector sensor data — Edison wipes its "ears" clean, like erasing a whiteboard, so old sounds don't confuse it.

Forever — this is a loop that never stops. Everything inside it keeps repeating, again and again, for as long as the program runs.

Wait until clap detected — Edison just stands still and listens. It won't do anything else until it hears a clap.

Clear clap detector sensor data (again) — as soon as it hears that first clap, it wipes its ears clean again, so it's ready to listen for a second clap without the first one confusing it.

Wait 0.2 seconds — Edison pauses for a fifth of a second. This gives your hands time to clap a second time.

If clap detected... then... else... — this is the big decision. Edison asks one question: "Did I hear a second clap in that little pause?"

- If yes → it goes forwards 30 cm at speed 5. One path.

- If no → it turns right 90 degrees at speed 5. The other path.

Only one of these two things happens each time — never both. That's what makes it an if/else: it's a fork in the road, not a checklist.

Back to the top of forever — after moving or turning, Edison loops back up to "wait until clap detected" and starts listening all over again.

Clear clap detector sensor data (the very last block) — this sits outside the forever loop. Since forever never actually finishes, Edison never reaches this block while the program is running.

### The two important gotchas

No forever loop → no repeating. If you took the forever block away, Edison would run the if/else exactly one time, the instant the program starts. Since no one has clapped yet, "clap detected" is false immediately, so it falls straight into else and just turns 90 degrees — then the whole program is done. It can't wait around for a first clap because there's nothing telling it to keep checking.

No wait → no time to clap twice. Computers check things incredibly fast, way faster than a human can clap twice in a row. Without that little wait 0.2 sec pause, Edison would check for the second clap before your hands even finish moving apart from the first clap. The wait is what gives your second clap a chance to be heard.

Extension

### The 3-clap version

Same skeleton as before, just one more layer added onto the existing if/else:

- clear sensor → forever

- wait until clap detected (clap 1) → clear sensor → wait 0.2 sec

- if clap detected again (clap 2)?

  - then → clear sensor → wait 0.2 sec → if clap detected a 3rd time?

    - then (3 claps) → beep + flash LED

    - else (2 claps) → forwards 30 cm at speed 5

  - else (1 clap) → turn right 90° at speed 5

- clear sensor → loop back to top

The trick: your old if/else didn't disappear — it just moved inside the "then" branch of a new outer if/else. Every extra clap you want to count means nesting one more if/else one level deeper, and giving Edison one more clear sensor + wait 0.2 sec pair to catch that next clap.

### The 4-clap version (harder mode)

Same idea, one level deeper again:

- clap 1 → wait/listen

- if clap 2? → if clap 3? → if clap 4?

  - then (4 claps) → your special action

  - else (3 claps) → beep + flash

  - else (2 claps) → forwards

- else (1 clap) → turn

By 4 claps you've got an if buried three levels inside another if inside another if — technically fine, but genuinely annoying to read and edit. This is the point where a clap counter variable is the smarter build: make a variable called claps, add 1 to it every time wait until clap detected fires, and reset it to 0 whenever there's a pause longer than, say, 1 second with no clap. Then one single check — "is claps equal to 4?" — replaces the whole nested pyramid.

That picture shows the "happy path" (every clap heard in time) for all three programs side by side. A couple of things to notice comparing the columns:

- Each program is the previous one with exactly one more if clap N? then ... else ... bolted onto the bottom of the last "then". Nothing else changes.

- Before every new if, there's a clear sensor + wait 0.2 sec pair (left off the picture above to keep it readable — it sits right before each "if clap N?" box).

- Every "yes" box also has an "else" hanging off it that drops back down to the shorter program's action — that's the part not drawn here, since it's identical to what the very first diagram showed you, just one level deeper each time.

So the 4-clap program, in full, reads like: clap 1 → if clap 2 → if clap 3 → if clap 4 → special action, else beep+flash, else forwards, else turn. Four nested nesting levels, four different possible outcomes, one program.

TEST

2-clap program:

- clap — single clap, then wait.  (turn 90 degree)

- clapclap, ~0.2 sec apart (move forward)

3-clap program:

- clap — single clap, then wait.

- clapclap, ~0.2 sec apart — this only exercises the outer if (clap 1 vs clap 2), same as the 2-clap test.

- clap ~0.2 sec clap ~0.5 sec clap — first gap stays short , and only the second gap widens to ~0.5 sec . So it's uneven spacing, not clap...clap...clap all equally 0.5 apart: clap-clap close together, then a slightly longer beat before the third clap.

# Line tracing

Check if your sensor mapping is working. If yes, you can now decode what's happening:

- C = non-reflective = off the orange line (probably reading the cardboard as non-reflective, since orange marker doesn't behave like a true white background) → triggers "turn robot left"

- G = reflective = on the orange line → triggers "turn robot right"

So the sensor genuinely can tell the difference between the two surfaces. The "moves forward" and "rotates in circles" descriptions are just what those two motor commands look like in practice:

C / turn left → looks like "moving forward" A single-wheel-speed-difference "turn" at low speed often looks more like a gentle forward drift than a sharp turn, especially if you're holding the robot rather than watching it drive freely.

G / turn right → looks like "rotating in circles" This is the real tell. If you're holding the robot roughly stationary over the orange mark while testing, and the sensor keeps reading reflective the whole time, the event just keeps re-firing "turn robot right" over and over with nothing to interrupt it — so it spins in place. That's not a bug, that's exactly what a sustained turn command does when there's no line edge for it to cross back over.

What this tells you: the logic and sensor detection are both fine. What you haven't tested yet is actual line-following behavior — quick alternation between left/right as the robot crosses back and forth over a line edge while driving forward. Holding it stationary over one spot can't show you that.

Next step: put the "set both motors to forwards" block back in properly (or leave it, since it's already there before the forever loop) and actually let the robot drive across your drawn orange line freely on the cardboard, rather than hand-holding it in place. Watch for it to wobble — quick left-right corrections — as it crosses the line, rather than doing a full stationary spin. If it still just spins in a full circle once it hits the line, the turn speed/duration may be too aggressive relative to line width, and you'd want to reduce turn speed so it corrects more gently and re-crosses back off the line sooner.